# DNS Covert Channel Analysis

Column-to-Technique Mapping (Based on Your Dataset)

## 1️⃣ DNS Query Domain (Payload Hiding)

### 📌 Dataset Columns Used

- dns_domain_name

- dns_subdomain_name_length

- dns_domain_name_length

- dns_top_level_domain

- dns_second_level_domain

### Normal (Baseline Traffic)

Columns

- dns_domain_name

- dns_domain_name_length

Example

dns_domain_name = google.com
dns_domain_name_length = 10
dns_subdomain_name_length = 0

Why Normal

- Human-readable

- Short length

- No encoded payload

### Header Manipulation (Covert Technique)

Columns

- dns_domain_name

- dns_subdomain_name_length

- dns_domain_name_length

Example

dns_domain_name = 15331.RVEVCNEH726ZS4L2QEDJEIK.api.daumel.xyz
dns_subdomain_name_length = 22
dns_domain_name_length = 55

Why Covert

- Payload embedded in QNAME

- Long subdomain labels encode data

- Classic DNS tunneling

✅ Primary column proving header manipulation:

dns_domain_name

### Time Obfuscation

Columns

- dns_domain_name

Example

dns_domain_name = google.com

Important

- Domain itself looks normal

- Covert behavior is NOT in this column

➡ revealed via timing columns (next section)

### Flow Bending

Columns

- dns_domain_name

- flow_id

Example

flow_id = 10231
dns_domain_name = part1.example.com

flow_id = 10232
dns_domain_name = part2.example.com

Why Covert

- Payload split across multiple DNS queries

- Same base domain, different subdomains

## 2️⃣ Character-Level Manipulation (Header Manipulation)

### 📌 Dataset Columns Used

- numerical_percentage

- character_distribution

- character_entropy

- uni_gram_domain_name

- bi_gram_domain_name

- tri_gram_domain_name

### Normal

numerical_percentage = 0.05
character_entropy = 3.1

### Header Manipulation

numerical_percentage = 0.42
character_entropy = 4.8

Why

- Encoded binary → base32/base64

- High randomness

✅ Strongest indicators in your dataset

- character_entropy

- numerical_percentage

## 3️⃣ TTL-Based Manipulation (Header Manipulation)

### 📌 Dataset Columns Used

- distinct_ttl_values

- ttl_values_min

- ttl_values_max

- ttl_values_mean

- ttl_values_mode

- ttl_values_median

### Normal

distinct_ttl_values = 1
ttl_values_mean = 300

### Header Manipulation

distinct_ttl_values = 5
ttl_values_min = 1
ttl_values_max = 20

Why

- Disables caching

- Forces communication with C2

## 4️⃣ Timing Behavior (Time Obfuscation)

### 📌 Dataset Columns Used

- timestamp

- duration

- packets_rate

### Normal

timestamp gaps = irregular
packets_rate = low, bursty
duration = short

### Time Obfuscation

timestamp gaps = consistent / jittered
packets_rate = artificially steady
duration = prolonged

Why

- Data encoded in inter-arrival times

- Evades payload inspection

✅ Primary evidence column

- timestamp

- packets_rate

## 5️⃣ Packet Size Consistency (Header + Time)

### 📌 Dataset Columns Used

- total_bytes

- sending_bytes

- receiving_bytes

- mean_packets_len

- standard_deviation_packets_len

- coefficient_of_variation_packets_len

### Normal

mean_packets_len = variable
coefficient_of_variation_packets_len = high

### Covert

mean_packets_len ≈ constant
coefficient_of_variation_packets_len = low

Why

- Encoded payloads generate uniform packets

## 6️⃣ Flow Bending (Traffic-Level Covert Channel)

### 📌 Dataset Columns Used

- flow_id

- src_ip

- dst_ip

- src_port

- dst_port

- duration

### Normal

1 flow → 1 DNS query → 1 response

### Flow Bending

Same src_ip + dst_ip
Multiple flow_id
Split payload across flows

Why

- Harder to detect per-flow

- Payload reconstruction needed

## 7️⃣ Resource Record Abuse (Header Manipulation)

### 📌 Dataset Columns Used

- ans_resource_record_type

- ans_resource_record_class

- distinct_A_records

### Normal

ans_resource_record_type = A
distinct_A_records = 1

### Header Manipulation

ans_resource_record_type = TXT / CNAME
distinct_A_records > 1

Why

- TXT/CNAME can carry hidden data

## 8️⃣ Label (Ground Truth)

### 📌 Dataset Column

- labe

labe = 0 → Normal
labe = 1 → Covert

Used for:

- ML training

- Evaluation

- Validation

## ✅ Final Mapping Summary (Very Important)

| Covert Technique | Main Dataset Columns |
| --- | --- |
| Normal Traffic | dns_domain_name, character_entropy, timestamp |
| Header Manipulation | dns_domain_name, character_entropy, numerical_percentage, ttl_*, ans_resource_record_type |
| Time Obfuscation | timestamp, packets_rate, duration |
| Flow Bending | flow_id, src_ip, dst_ip, dns_domain_name |

If you want, next I can:

- 🔹 Convert this into a Methodology section (thesis-ready)

- 🔹 Create a figure explaining covert DNS channels

- 🔹 Map these columns to ML feature groups

Just tell me what you need next.