ICMP Antiforensic technique

## 1️⃣ Time Obfuscation in ICMPv6

(Using the Time feature)

### What time obfuscation means

Time obfuscation hides covert communication by controlling when ICMP packets are sent, instead of changing their content. Information is encoded in packet delays, not payload.

### How manipulation is performed

In ICMPv6 covert channels:

- A sender intentionally adds delays between packets

- Different delay values represent different symbols:

  - Short delay → 0

  - Long delay → 1

- Delays may be:

  - Fixed (e.g., 100 ms vs 500 ms)

  - Randomized within ranges to avoid detection

Example:

Packet 1 at t = 1.00s

Packet 2 at t = 1.10s → 0

Packet 3 at t = 1.60s → 1

### How your dataset shows this

#### Relevant column:

- Time

Although you don’t have explicit inter-arrival features, you can compute:

ΔTime = Time(i) − Time(i−1)

Indicators of manipulation:

- Repeating delay patterns

- Non-human-like ICMP timing

- Consistent timing gaps between Echo Requests

### Why this is antiforensic

- Payload remains clean

- Header fields look normal

- Signature-based IDS fails

- Only temporal analysis reveals the covert channel

✅ Conclusion:
Time obfuscation is performed by encoding data in ICMPv6 packet timing, visible through irregularities in the Time column.

## 2️⃣ Header Manipulation in ICMPv6

(Main antiforensic surface in your dataset)

Header manipulation hides information inside legitimate protocol fields, making packets look normal but semantically abnormal.

### 2.1 Hop Limit Manipulation

Column: Hop Limit

#### How manipulation is performed

- Instead of decrementing naturally, attackers:

  - Fix hop limits at unusual values

  - Encode data using hop limit numbers

    - Even = 0

    - Odd = 1

Example:

Hop Limit = 64 → 0

Hop Limit = 65 → 1

#### Why it works

- Firewalls rarely inspect hop-limit entropy

- ICMP doesn’t require strict hop-limit values

### 2.2 Packet Length Manipulation

Column: Length

#### How manipulation is performed

- Payload size is adjusted deliberately

- Length encodes covert data

Example:

Length = 60 bytes → 0

Length = 84 bytes → 1

#### Why it works

- Echo payload length is flexible

- IDS often allows variable ICMP sizes

### 2.3 DSCP & ECN Manipulation

Columns: DSCP, ECN

#### How manipulation is performed

- DSCP bits (6 bits) are repurposed

- ECN bits carry hidden flags

- Values change even when QoS is unused

Example:

DSCP = 0 → normal

DSCP = 46 → covert signal

#### Why it works

- QoS fields are rarely enforced

- Looks like routing metadata, not data

### 2.4 IPv6 Flow Label Manipulation

Column: FlowLabel

#### How manipulation is performed

- Flow label should be random or constant per flow

- Attackers encode data by:

  - Sequential flow labels

  - Binary patterns

Example:

FlowLabel = 0x00001 → 0

FlowLabel = 0x00003 → 1

#### Why it works

- Flow labels are poorly monitored

- Large entropy space (20 bits)

### 2.5 Next Header Manipulation

Column: Next Header

#### How manipulation is performed

- Field may be altered inconsistently

- Used to confuse protocol parsing

- Sometimes toggled between valid values

#### Why it works

- Parsing ambiguity disrupts DPI systems

### 2.6 ICMPv6 Type & Code Manipulation

Columns: ICMPv6 Type, ICMPv6 Code

#### How manipulation is performed

- Rare or unnecessary types are used

- Code values changed without semantic meaning

- Sequence of types encodes data

Example:

Type 128 → 0

Type 129 → 1

#### Why it works

- ICMP allows many valid types

- Most networks don’t restrict them

### 2.7 ICMPv6 Checksum Manipulation

Column: ICMPv6 Checksum

#### How manipulation is performed

- Payload is altered, checksum recalculated

- Patterned checksum values may appear

- Used to verify hidden payload integrity

#### Why it works

- Correct checksum avoids packet drops

- IDS rarely inspects checksum entropy

### 2.8 Info Field Manipulation

Column: Info

#### How manipulation is performed

- ID and sequence numbers are modified

- Echo sequence field encodes data

- Non-incremental sequences indicate covert use

Example:

seq=1 → 0

seq=9 → 1

### Header Manipulation Summary

✅ Your dataset strongly supports header manipulation analysis
These fields are classic ICMP antiforensic carriers.

## 3️⃣ Flow Bending in ICMPv6

(Reconstructed using basic features)

### What flow bending means

Flow bending hides covert communication by breaking normal flow behavior, without touching payload.

### How manipulation is performed

Attackers:

- Send ICMP packets out of order

- Fragment communication across flows

- Reset flows intentionally

- Change destinations mid-communication

### How your dataset represents it

#### Relevant columns:

- Time

- Source

- Destination

- Protocol

- Host

Using these, flows can be reconstructed as:

(Source, Destination, Protocol)

Indicators:

- Sudden flow breaks

- Irregular ICMP request-reply pairing

- Frequent source/destination switching

- Non-linear packet timing within same host pair

### Why this is antiforensic

- Avoids long detectable flows

- Confuses flow-based ML models

- Mimics legitimate ping noise

## Final Mapping

| Technique | Columns Used | Manipulation Mechanism |
| --- | --- | --- |
| Time Obfuscation | Time | Encoding data in packet delays |
| Header Manipulation | Hop Limit, Length, DSCP, ECN, FlowLabel, Next Header, ICMPv6 Type, Code, Checksum, Info | Hiding data inside protocol headers |
| Flow Bending | Time, Source, Destination, Host | Breaking normal flow structure |